How to Interpret Status LEDs on Cisco Meraki MR45 / MR46 Access Points

KB003    Category: Infrastructure    54 views

[Applies to: BlueStar Airlines, Inc. — IT Infrastructure and Network Teams]

Summary

This article explains the LED color and blink patterns on Cisco Meraki MR45 and MR46 wireless access points (APs) deployed across BlueStar Airlines terminals, gate areas, crew lounges, and office buildings. Technicians can use this guide to assess AP health during on-site visits without requiring Meraki Dashboard access.

Details

Overview

The MR45 and MR46 each have a single multicolor status LED on the underside of the unit. The LED color and blink behavior reflects the AP's boot state, cloud connectivity, client load, and fault status. Healthy baseline: solid white or solid blue (depending on firmware version) with cloud connected and clients associated.

LED States and Meanings

| LED Color / State | Meaning |
| --- | --- |
| Solid white | AP is online, connected to Meraki Cloud, and serving clients normally. |
| Solid blue | AP is online and operational (alternate firmware display — same as white). |
| Blinking white (slow, ~1 s) | Firmware upgrade in progress — do NOT power off. |
| Blinking orange (slow) | AP is powered but cannot reach Meraki Cloud — check uplink and VLAN tagging. |
| Blinking orange (fast) | LED self-test pattern — normal during first 30 seconds after power-on. |
| Solid orange | AP is online and operational at reduced capacity — one radio band is disabled. |
| Blinking green | AP is associated to cloud but no clients are connected (idle/low-traffic). |
| Off | No power — check PoE switch port, PoE budget, and cable. |
| Cycling colors (rainbow) | AP is performing a factory reset — triggered by holding the reset button for 10 s. |

Quick Health Check Procedure

- Confirm LED is solid white or solid blue — this is the only fully healthy state.

- If blinking orange (slow): verify the AP's uplink port on the IDF switch is up and the AP VLAN (VLAN 10 at BlueStar) is tagged correctly.

- If blinking white: firmware upgrade is in progress — wait up to 15 minutes; do not interrupt power.

- If solid orange: AP is operating in single-band fallback mode — check Dashboard for radio alerts.

- If off: test PoE delivery with a PoE tester; confirm the switch port is in Access mode on VLAN 10.

Common Scenarios & Troubleshooting

| Scenario | Likely Cause | Action |
| --- | --- | --- |
| AP blinking orange after network change | VLAN or ACL misconfiguration blocking cloud | Verify VLAN tagging and confirm AP can reach 209.206.50.0/20 (Meraki cloud) on UDP 7532. |
| AP solid orange, reduced coverage | Single radio band failure | Check Meraki Dashboard → Access Points → Radio Settings for band errors. |
| AP off after patch panel re-shuffle | PoE port not enabled or wrong port patched | Verify patch and confirm "show power inline" on switch port shows powered. |
| Clients complain of drops, LED white | RF interference or channel saturation at gate | Check channel utilization in Meraki Dashboard; consider manual channel assignment. |
| AP cycling through colors | Factory reset in progress (accidental button press) | Wait for reset to complete (~5 min), then re-claim in Dashboard. |

Notes

BlueStar's Meraki network uses Organization "BlueStar-Prod." All APs are licensed under the Enterprise license. LED behavior may differ slightly between MR45 and MR46 on firmware versions below MR 29.x — upgrade to MR 30.x or later via Dashboard → Firmware Upgrades to align LED behavior with this guide.

Support Information

| Name | Role | Email | Phone |
| --- | --- | --- | --- |
| IT Helpdesk | General IT Support | helpdesk@bluestar.com | 1-800-742-9300 |
| Patel, Ravi | Network Infrastructure Engineer | rpatel@bluestar.com | x9320 |
| NOC (24/7) | Network Operations Center | noc@bluestar.com | 1-800-742-9322 |

Related Articles

• KB004 – APC Smart-UPS Battery Status and LCD Guide

• KB006 – Submitting an IT Support Request via the Self-Service Portal

• Meraki Dashboard: https://dashboard.meraki.com (requires BlueStar SSO)